Quantitative Lipidomics (Coral Tissue)

Sample Prep

Coral tissue samples (N=98) were prepared in three separate batches (31+31+36). Frozen samples were first thawed at 4 °C for 60 min and afterwards were homogenized in a Bullet Blender with zirconia beads for 45 minutes, and then the mixtures were transferred to glass culture tubes. The following were added to the samples in glass tubes: 25 μL of a mixture containing 68 isotope labeled internal standard lipids, 575 μL of methyl-tert-butyl ether (MTBE) and 160 μL of methanol (MeOH). Samples were shaken in the monophase and incubated for 30 min at room temperature. 200 μL of H2O was added afterwards and samples were centrifuged at 2,500 g for 3 min at room temperature (two distinguished liquid layers were generated). The upper layer was transferred to a new glass vial using a glass Pasteur pipette. The lower layer was used to perform a second extraction by adding 300 µL MTBE, 100 µL MeOH and 100 µL of H2O and performing shaking and centrifugation as in the first extraction. The recovered upper layer from the second extraction was added to the glass tube containing the recovered layer from the first extraction, and the combined solution was dried under nitrogen (the protein precipitate pallet from the lower layer was used for a BCA assay needed for data normalization). The dried samples were reconstituted in 250 μL of 10 mM ammonium acetate in 50:50 methanol:dichloromethane. At this point the samples were ready for mass spectrometry analysis. NOTICE: The protein precipitate pallets were used to determine the total protein amount in each sample using a BCA assay (needed for data normalization).

Mass Spectrometry Data Acquisition

Lipid species were measured using a Sciex Lipidyzer system as described elsewhere.1,2 The Sciex Lipidyzer mass spectrometry platform consisted of Shimadzu Nexera X2 LC-30AD pumps, a Shimadzu Nexera X2 SIL-30AC autosampler kept at 10 oC and a Sciex QTRAP 5500 mass spectrometer equipped with ESI source coupled with SelexION for differential mobility spectrometry (DMS). 1-propanol was used as the chemical modifier for the DMS. Samples were introduced to the mass spectrometer by flow injection analysis at 8 uL/min. Each sample was injected twice, first with the DMS ON (PC, PE, LPC, LPE, PG, PI, PS and SM), and then with the DMS OFF (CE, CER d18:0, CER d18:1, DAG, FFA, HexCER, LacCER, LPG, LPI, LPS, PA and TAG). 1,561 lipid species (plus 68 spiked isotope-labeled lipid standards) across 20 lipid classes/sub-classes were targeted using multiple reaction monitoring (MRM) and positive/negative polarity switching electro-spray ionization. SM, DAG, CE, CER d18:0, CER d18:1, HexCER, LacCER, LPG, LPI, LPS, PA, LPE, LPC and TAG were ionized in positive and PE, PG, PI, PS, PC, and FFA in negative ionization mode, respectively. Data were acquired using Analyst 1.6.3 and processed with an open-source Shotgun Lipidomics Assistant v4.1.2 software.1 Up to 1,122 lipid species were measured across the study. For quality control, a pooled human serum sample (in-house quality control) was run at the beginning and at the end of each batch, respectively. The median inter-batch CV was 8.3%, and 11 of 20 measured lipid classes/sub-classes had a CV value of under 10%. The final data contained absolute concentrations of each lipid species based on the known concentrations of the spiked isotopically labeled internal standard lipids.

- Zhu W, Lusk JA, Pascua V, Djukovic D, Raftery D. Combination of low glucose and SCD1 inhibition impairs cancer metabolic plasticity and growth in MCF-7 cancer cells: a comprehensive metabolomic and lipidomic analysis.

Metabolomics. 2024 Oct 5;20(5):112.

- Baolong Su, Lisa F. Bettcher, Wei-Yuan Hsieh, Daniel Hornburg, Mackenzie J. Pearson, Niek Blomberg, Martin Giera, Michael P. Snyder, Daniel Raftery, Steven J. Bensinger and Kevin J. Williams. A DMS Shotgun Lipidomics Workflow Application to Facilitate High-Throughput, Comprehensive Lipidomics. J. Am. Soc. Mass Spectrom. 2021, 32, 2655−2663